OCI GOVERNMENT CLOUD

# FastConnect and Existing DRG
to CD3 Network

Private PuTTY/SSH Access from an On-Premises Jumpbox

Implementation Runbook

Environment: EBLZ / SCCA Landing Zone / CD3-managed VCNs

Region: us-langley-1

Version: 1.1 | Date: September 3, 2026

Purpose Connect an existing FastConnect private virtual circuit and DRG to CD3-managed OCI networks so approved administrators can use PuTTY to reach OCI Linux instances by private IP, without OCI Bastion, public IPs, or portal-based SSH.

# 1. Executive Summary

The existing FastConnect DRG should remain the enterprise on-premises connectivity gateway. CD3-created workload and management VCNs attach to that same upgraded DRG. FastConnect does not attach directly to an individual CD3 VCN.

Critical control Export the existing DRG, route tables, attachments and rules into the CD3 workbook and Terraform state before modifying the network. Do not represent the existing DRG as a new resource, and do not create a second DRG solely for the CD3 VCN.

## Target traffic paths

| Mode | Traffic path | Use |
| --- | --- | --- |
| SCCA preferred | On-prem jumpbox -> CPE -> FastConnect -> existing DRG -> VDSS inspection hub/firewall -> DRG -> CD3 spoke -> OCI VM | Production when centralized inspection is required |
| Direct temporary | On-prem jumpbox -> CPE -> FastConnect -> existing DRG -> CD3 VCN -> OCI VM | Controlled interim testing only when security architecture permits |

## Success criteria

•  The FastConnect private virtual circuit is provisioned and BGP is up.

•  The existing DRG learns the approved on-premises jumpbox prefix.

•  The on-premises router learns the approved CD3 management prefix.

•  Forward and return traffic follow the same approved inspection path.

•  Only approved jumpbox addresses can reach TCP/22 on OCI management VNICs.

•  PuTTY authenticates with the correct username and private key against the OCI VM private IP.

# 2. Planning Inputs

Replace the examples below with approved production values before implementation.

| Item | Example value | Production value |
| --- | --- | --- |
| On-premises network | 172.20.0.0/16 | ____________________________ |
| Approved jumpbox | 172.20.10.25/32 | ____________________________ |
| CD3 VCN | 10.50.0.0/16 | ____________________________ |
| OCI management subnet | 10.50.10.0/24 | ____________________________ |
| OCI target VM | 10.50.10.20 | ____________________________ |
| Existing DRG OCID | ocid1.drg... | ____________________________ |
| Virtual circuit OCID | ocid1.virtualcircuit... | ____________________________ |

Addressing prerequisite On-premises and OCI CIDR blocks must not overlap. Resolve overlaps before attaching the VCN or advertising routes.

# 3. Pre-Change Validation

## 3.1 Existing FastConnect and DRG

1.  Open Networking -> Customer connectivity -> Dynamic Routing Gateways and select the existing FastConnect DRG.

2.  Confirm the DRG is upgraded (DRGv2).

3.  Confirm the private virtual circuit lifecycle state is PROVISIONED.

4.  Confirm both BGP sessions are UP where redundant BGP is configured.

5.  Record the virtual-circuit attachment and its associated DRG route table.

6.  Verify that the DRG imports the approved on-premises jumpbox prefix.

7.  Capture the current DRG routes and attachment associations for rollback evidence.

| Required route | Expected next hop |
| --- | --- |
| On-premises/jumpbox prefix | FastConnect virtual-circuit attachment |
| CD3 VCN or management prefix | CD3 VCN attachment or VDSS hub attachment |

## 3.2 Existing controls

•  Confirm the approved SCCA flow: direct DRG-to-spoke or mandatory VDSS inspection.

•  Confirm the on-premises firewall permits TCP/22 only from approved jumpboxes.

•  Confirm the target image has SSH enabled and the correct public key installed.

•  Confirm the network-administrator group already has sufficient virtual-network permissions. At the current IAM policy-object limit, append required statements to an existing consolidated policy rather than creating another policy object.

# 4. Bring Existing Networking Under CD3 Management

Use CD3's Export Resources workflow before making changes. The objective is to capture the live network in the workbook and Terraform state, not recreate it.

1.  Open the appropriate CD3 blank workbook.

2.  Run setupOCI and select Export Resources.

3.  Export the existing VCNs, DRG, attachments, route tables, route distributions, security lists and NSGs.

4.  Verify the existing DRG appears in the DRGs tab.

5.  Verify live DRG rules appear in DRGRouteRulesInOCI.

6.  Verify the existing FastConnect virtual-circuit attachment is preserved.

7.  Add the CD3 VCN-to-existing-DRG attachment in the correct networking tab/configuration.

8.  Generate Terraform and inspect the plan. It must show the intended attachment and route changes only.

9.  Stop if the plan proposes replacing the DRG, virtual circuit, existing attachments or unrelated routes.

No duplicate gateway A DRG can serve multiple VCNs, while a VCN can attach to only one DRG. If the CD3 VCN is already attached to a different DRG, resolve that architecture before proceeding.

# 5. Attach the CD3 VCN to the Existing DRG

| Attachment setting | Required value |
| --- | --- |
| DRG | Existing FastConnect DRG |
| Network type | VCN |
| VCN | CD3 workload or management VCN |
| DRG route table | Approved on-prem/CD3 or spoke route table |
| VCN route type | Approved VCN CIDRs or subnet CIDRs |
| Lifecycle state | ATTACHED |

For advanced transit routing, associate the correct VCN ingress route table with the attachment. Do not use a default table when it would permit inspection bypass or broader reachability than approved.

# 6. Configure DRG Routing

## 6.1 Traffic entering from FastConnect

The route table associated with the FastConnect virtual-circuit attachment controls traffic entering the DRG from on-premises.

### Direct temporary route

| Destination | Next-hop attachment |
| --- | --- |
| 10.50.0.0/16 | CD3 VCN attachment |

### SCCA inspected route

| Destination | Next-hop attachment |
| --- | --- |
| 10.50.0.0/16 | VDSS inspection-hub VCN attachment |

The route can be learned through an approved import distribution or configured statically. In the inspected design, the VDSS firewall forwards permitted traffic back through the DRG toward the CD3 spoke.

## 6.2 Return route from the CD3 subnet

| Location | Destination | Target |
| --- | --- | --- |
| OCI VM subnet route table | 172.20.0.0/16 | DRG |
| DRG table used by CD3 VCN attachment | 172.20.0.0/16 | FastConnect virtual-circuit attachment |

Use the specific approved on-premises CIDR instead of 0.0.0.0/0. The DRG return route is ordinarily learned dynamically from FastConnect BGP.

## 6.3 Route advertisement to on-premises

•  Advertise the approved CD3 management subnet, or the wider VCN prefix only if required.

•  Confirm the CPE accepts the advertised prefix and installs it in the enterprise route table.

•  Confirm no more-specific enterprise route sends the CD3 prefix to an internet or backup VPN path.

•  Confirm FastConnect remains the preferred route according to the approved BGP policy.

# 7. Configure SSH Security Controls

Attach a management NSG to the target VM VNIC and use the narrowest approved source.

| Field | Required value |
| --- | --- |
| Direction | Ingress |
| State | Stateful |
| Source type | CIDR |
| Source | 172.20.10.25/32 or approved jumpbox subnet |
| Protocol | TCP |
| Source port | All |
| Destination port | 22 |
| Description | SSH from approved on-premises jumpbox |

Prohibited rule Do not allow TCP/22 from 0.0.0.0/0. FastConnect private access does not require a public IP or internet-facing SSH rule.

## Additional enforcement points

•  The VDSS firewall must allow the same jumpbox-to-management-subnet TCP/22 flow.

•  The on-premises firewall must permit the outbound connection and its response.

•  Security lists must not contradict the intended access model.

•  Endpoint protection and local Windows firewall policy must permit PuTTY.

•  Stateful NSG rules allow the response flow, but return routing remains mandatory.

# 8. Verify the OCI Linux Host

From an approved administration path, verify OpenSSH and the host firewall.

sudo systemctl status sshd
sudo ss -lntp | grep ':22'
sudo firewall-cmd --list-services
sudo firewall-cmd --list-ports

If SSH is approved but not enabled:

sudo systemctl enable --now sshd
sudo firewall-cmd --permanent --add-service=ssh
sudo firewall-cmd --reload

| Image | Typical default username |
| --- | --- |
| Oracle Linux | opc |
| Red Hat-compatible OCI image | opc |
| Ubuntu | ubuntu |

Confirm the public key matching the administrator's approved PuTTY private key is present in the target account's authorized_keys file. Protect private keys according to organizational policy.

# 9. Test from the Windows Jumpbox

## 9.1 TCP connectivity

Test-NetConnection 10.50.10.20 -Port 22

Expected result:

TcpTestSucceeded : True

## 9.2 PuTTY session

| PuTTY field | Value |
| --- | --- |
| Host Name | OCI VM private IP, for example 10.50.10.20 |
| Port | 22 |
| Connection type | SSH |
| Username | opc or ubuntu, as appropriate |
| Private key | Approved .ppk file |
| Saved session | Example: OCI-CD3-MSAS-DEV-01 |

# 10. Network Path Analyzer Validation

Before production use, run a bidirectional Network Path Analyzer test.

| Parameter | Value |
| --- | --- |
| Source | On-premises jumpbox IP |
| Source classification | This IP address is an on-premises endpoint |
| Destination | OCI target VM VNIC/private IP |
| Protocol | TCP |
| Destination port | 22 |
| Direction | Bidirectional |

Review each hop. Resolve every red or indeterminate route/security result before approving production access.

# 11. Detailed Step-by-Step Implementation Procedure

Execute the following tasks in order. Record the actual OCIDs, CIDRs, route-table names and test results in the change record. Do not proceed past a failed validation gate.

## Task 1 - Record the approved network values

1.  Record the existing DRG name, compartment and OCID.

2.  Record the FastConnect private virtual-circuit name, OCID and attachment name.

3.  Record the on-premises jumpbox IP or approved jumpbox subnet.

4.  Record the CD3 VCN CIDR, OCI management subnet CIDR and target VM private IP.

5.  Confirm the on-premises and OCI CIDRs do not overlap.

6.  Confirm whether the security architecture requires VDSS firewall inspection.

Gate 1 - Addressing and path approval Proceed only after the network owner confirms non-overlapping CIDRs and identifies either the SCCA inspected path or an explicitly approved temporary direct path.

## Task 2 - Verify FastConnect and BGP in the OCI Console

1.  Sign in to the OCI Government Cloud Console for us-langley-1.

2.  Open Networking -> Customer connectivity -> FastConnect.

3.  Select the existing private virtual circuit.

4.  Confirm Lifecycle state is PROVISIONED.

5.  Confirm BGP session state is UP for every required BGP session.

6.  Record the attached DRG and BGP IPv4 addresses.

7.  Open Received routes and confirm the approved on-premises jumpbox prefix is present.

8.  Open Advertised routes and capture the current OCI prefixes before making changes.

Gate 2 - FastConnect health Do not change DRG routing while the private virtual circuit is not provisioned or required BGP sessions are down. Correct the FastConnect/CPE condition first.

## Task 3 - Export the existing network into CD3

1.  Back up the current CD3 workbook, generated Terraform and state files.

2.  Start the approved CD3 setupOCI environment.

3.  Select Export Resources -> Network.

4.  Choose us-langley-1 and the network compartments containing the existing DRG, FastConnect attachment and CD3 VCNs.

5.  Export VCNs, DRGs, VCN attachments, DRG route tables, route distributions, route rules, subnet route tables, security lists and NSGs.

6.  Open the exported workbook and locate the existing DRG in the DRGs tab.

7.  Verify the live DRG rules in DRGRouteRulesInOCI and the CD3 VCN/subnets in their networking tabs.

8.  Generate or refresh the CD3 Terraform state using the supported export workflow.

9.  Run a Terraform plan without proposed changes and confirm it does not attempt to create or replace the existing DRG, virtual circuit or existing attachments.

Gate 3 - Zero-change baseline The baseline plan must be understood before adding the new attachment. Stop if existing resources appear as new, missing, or scheduled for replacement.

## Task 4 - Attach the CD3 VCN to the existing DRG

Preferred method: represent the attachment in the CD3 workbook/Terraform configuration after the export. Use the manual Console steps below only for an approved manual implementation or validation.

1.  Open Networking -> Customer connectivity -> Dynamic Routing Gateways.

2.  Select the existing FastConnect DRG.

3.  Under Attachments, open Virtual Cloud Network attachments.

4.  Select Create VCN attachment.

5.  Enter a controlled name such as CD3-MSAS-DEV-DRG-ATTACHMENT.

6.  Select Current tenancy and choose the compartment containing the CD3 VCN.

7.  Select the CD3 VCN.

8.  Open Advanced options.

9.  Select the approved DRG route table for traffic entering the DRG from this VCN. Do not accept the default blindly.

10.  For advanced transit routing, select the approved VCN ingress route table associated with the attachment.

11.  Select the approved route type: VCN CIDRs or the required subnet CIDRs.

12.  Create the attachment and wait until Lifecycle state is ATTACHED.

Single-DRG restriction If the VCN is already attached to another DRG, stop. A VCN can have only one DRG attachment. Do not detach the existing DRG without an approved migration and rollback plan.

## Task 5 - Configure traffic entering the DRG from FastConnect

1.  Inside the existing DRG, open DRG route tables.

2.  Identify the route table associated with the FastConnect virtual-circuit attachment.

3.  Create or select the approved route table for traffic entering from on-premises, for example DRG-RT-FROM-ONPREM.

4.  Add the CD3 destination prefix using the correct branch below.

5.  Save the route table.

6.  Open the virtual-circuit attachment, select Edit and associate DRG-RT-FROM-ONPREM if it is not already associated.

7.  Reopen the route table and verify the route is ACTIVE.

| Approved branch | Destination | Next hop |
| --- | --- | --- |
| SCCA production | CD3 VCN or management CIDR | VDSS inspection-hub VCN attachment |
| Temporary direct | CD3 VCN or management CIDR | CD3 VCN attachment |

Inspection warning For the SCCA path, do not add a more-specific direct route to the CD3 VCN. A more-specific route can bypass the VDSS firewall even when a broader route points to the inspection hub.

## Task 6 - Configure DRG return routing to FastConnect

Traffic entering the DRG from the CD3 VCN or VDSS hub must learn the on-premises prefix from the FastConnect attachment.

1.  Inside the DRG, open Route distributions and create or select the approved import route distribution for the CD3 or hub attachment route table.

2.  Add an import statement matching the specific FastConnect virtual-circuit attachment, or the approved attachment-type filter when organizational policy permits it.

3.  Save the import distribution.

4.  Open the DRG route table associated with the CD3 VCN attachment, or the VDSS hub attachment in the inspected design.

5.  Enable Import route distribution and select the approved distribution.

6.  Wait for route convergence.

7.  Open the DRG route rules and verify the approved on-premises prefix is ACTIVE with the FastConnect virtual-circuit attachment as the next hop.

Gate 4 - DRG bidirectional routes The FastConnect attachment table must resolve the OCI destination, and the CD3/hub attachment table must resolve the on-premises jumpbox prefix. Both directions are required.

## Task 7 - Add the OCI subnet return route

1.  Open Networking -> Virtual cloud networks and select the CD3 VCN.

2.  Open Subnets and select the subnet containing the target VM.

3.  Record the route table associated with that subnet.

4.  Open the route table and select Add route rules.

5.  Set Target type to Dynamic Routing Gateway.

6.  Set Destination type to CIDR block.

7.  Enter the approved on-premises or jumpbox CIDR, for example 172.20.0.0/16 or the narrower approved prefix.

8.  Select the existing DRG and save the rule.

9.  Confirm the route appears in the subnet's effective route table.

## Task 8 - Verify OCI route advertisement at the on-premises CPE

1.  Return to FastConnect -> private virtual circuit -> Advertised routes.

2.  Confirm OCI advertises the approved CD3 management prefix or inspection-hub prefix.

3.  On the CPE, verify the prefix is received over the FastConnect BGP neighbor.

4.  Verify the enterprise routing table selects FastConnect as the active next hop.

5.  Verify no overlapping or more-specific route sends the OCI prefix to the internet, an unintended VPN, or another data center.

6.  From the on-premises jumpbox, run tracert to the OCI private IP. Treat nonresponsive intermediate hops cautiously; use the route decision and TCP test as the primary evidence.

tracert 10.50.10.20
Test-NetConnection 10.50.10.20 -Port 22

## Task 9 - Create the SSH NSG rule

1.  Open Networking -> Virtual cloud networks -> Network Security Groups.

2.  Select the management NSG attached to the target VM VNIC, or create the approved management NSG through CD3.

3.  Open Security rules and select Add rules.

4.  Set Direction to Ingress and leave the rule stateful.

5.  Set Source type to CIDR.

6.  Enter the approved jumpbox /32 address or management subnet. Do not use 0.0.0.0/0.

7.  Set IP protocol to TCP, source port to All and destination port to 22.

8.  Add a change-ticket reference in the description and save.

9.  Open Compute -> Instances -> target instance -> Attached VNICs and verify that the management NSG is attached to the correct VNIC.

## Task 10 - Configure the intervening firewalls

| Enforcement point | Required rule |
| --- | --- |
| On-premises firewall | Approved jumpbox source -> OCI management CIDR, TCP/22 |
| VDSS firewall | Approved jumpbox source -> CD3 management CIDR, TCP/22, with symmetric return path |
| OCI NSG | Approved jumpbox /32 or subnet -> target VNIC, TCP/22 |
| Linux host firewall | Allow SSH service/TCP 22 from the approved path |

Enable logging at the on-premises and VDSS enforcement points for the test window. Avoid address translation unless it is part of the approved architecture; if NAT occurs, the OCI NSG must allow the translated source address actually observed at the VNIC.

## Task 11 - Verify SSH on the OCI VM

1.  Use an existing approved administration path to access the VM.

2.  Confirm sshd is active and listening on TCP/22.

3.  Confirm the Linux firewall allows the SSH service.

4.  Confirm the correct administrator public key is installed in the target account's authorized_keys file.

5.  Confirm the account name matches the image: commonly opc for Oracle Linux/RHEL or ubuntu for Ubuntu.

6.  Do not copy an unprotected private key to shared folders or the target VM.

sudo systemctl is-active sshd
sudo ss -lntp | grep ':22'
sudo firewall-cmd --list-services
sudo firewall-cmd --list-ports

## Task 12 - Configure and test PuTTY

1.  From the approved Windows jumpbox, run Test-NetConnection against the OCI VM private IP and port 22.

2.  Open PuTTY and enter the OCI VM private IP under Host Name.

3.  Set Port to 22 and Connection type to SSH.

4.  Under Connection -> Data, enter the approved auto-login username if required.

5.  Under Connection -> SSH -> Auth -> Credentials, select the approved .ppk private-key file.

6.  Return to Session and save the configuration using a controlled session name.

7.  Select Open, validate the server host-key fingerprint through an approved source, and accept it only after validation.

8.  Confirm the login succeeds without using a public IP, OCI Bastion or portal SSH.

Test-NetConnection 10.50.10.20 -Port 22

## Task 13 - Run positive and negative validation

1.  Run Network Path Analyzer from the on-premises jumpbox IP to the target VNIC on TCP/22 and select Bidirectional.

2.  Mark the jumpbox IP as an on-premises endpoint.

3.  Confirm the discovered path follows the approved direct or VDSS-inspected route.

4.  Repeat Test-NetConnection and PuTTY from the approved jumpbox and record success.

5.  From an unauthorized on-premises source, verify TCP/22 is denied.

6.  Review NSG, VDSS firewall and on-premises firewall logs for the expected source, destination and action.

7.  Confirm existing production routes and services remain healthy.

## Task 14 - Close the change

1.  Save the final CD3 workbook, Terraform plan/apply output and state backup.

2.  Export the final DRG route rules, route distributions, attachment associations, subnet route rule and NSG rule.

3.  Attach positive and negative test evidence to the change record.

4.  Record the final PuTTY target private IP, username and key owner without recording private-key material.

5.  Obtain network, security and application-owner acceptance.

6.  Monitor FastConnect BGP, DRG attachment drops and firewall denies during the post-change observation window.

# 12. Troubleshooting Matrix

| Observed result | Likely cause | Check |
| --- | --- | --- |
| CD3 prefix absent on CPE | DRG route is not exported through BGP | Virtual-circuit DRG table, export policy and CPE filters |
| On-prem prefix absent in DRG | CPE is not advertising it or import distribution excludes it | BGP advertised routes and DRG import distribution |
| Test-NetConnection times out | Routing, firewall, NSG or asymmetric path | Path Analyzer, route tables and firewall logs |
| TCP/22 connection refused | Host reached but sshd is stopped or host firewall rejects | systemctl, ss and firewall-cmd |
| Permission denied (publickey) | Wrong user, key or authorized_keys entry | Username, .ppk and installed public key |
| Firewall receives but spoke does not | Hub or DRG return route missing | VDSS ingress/egress and DRG tables |
| One-way connectivity | Missing or asymmetric return route | Subnet route, DRG route and CPE route |

# 13. Change and Rollback Plan

## Implementation order

1.  Export and preserve the live CD3/OCI network state.

2.  Validate non-overlapping CIDRs and approved SCCA traffic flow.

3.  Attach the CD3 VCN to the existing DRG.

4.  Apply DRG route-table and import-distribution changes.

5.  Apply the CD3 subnet return route.

6.  Update on-premises BGP filters and firewalls.

7.  Apply the narrow NSG rule for TCP/22.

8.  Run Path Analyzer and TCP tests.

9.  Test PuTTY authentication.

10.  Capture evidence and obtain acceptance.

## Rollback triggers

•  Terraform proposes replacement of the existing DRG, virtual circuit or unrelated attachments.

•  Existing FastConnect routes disappear or BGP resets unexpectedly.

•  Traffic bypasses the required VDSS inspection path.

•  Return routing becomes asymmetric.

•  Unapproved networks gain TCP/22 reachability.

•  Existing production VCN connectivity is degraded.

## Rollback actions

1.  Remove or disable the new on-premises advertisement/filter change.

2.  Restore the previous DRG route-table association and route distributions.

3.  Restore the previous CD3 subnet route table.

4.  Remove the new SSH NSG rule.

5.  Detach the CD3 VCN only if the attachment was newly created and no approved traffic depends on it.

6.  Re-run bidirectional validation and document restoration.

# 14. Acceptance Checklist

| Control | Evidence | Status |
| --- | --- | --- |
| FastConnect provisioned and BGP up | Virtual-circuit/BGP screenshot or CLI output | [ ] |
| Existing DRG preserved in CD3 state | Export workbook and Terraform state/plan | [ ] |
| Approved forward route | DRG route-table evidence | [ ] |
| Approved return route | Subnet and DRG route evidence | [ ] |
| Inspection path enforced where required | Path Analyzer and firewall evidence | [ ] |
| TCP/22 limited to approved jumpbox | NSG and firewall rule evidence | [ ] |
| Host SSH operational | sshd/listener output | [ ] |
| PuTTY private-IP connection successful | Connection test record | [ ] |
| Unauthorized source denied | Negative test record | [ ] |
| Rollback artifacts retained | Pre/post state and change record | [ ] |

# 15. Reference Documentation

Oracle: Attaching a VCN to a DRG

Oracle: Centralized transit routing through a network appliance

Oracle: Working with VCN route tables

Oracle: Network Path Analyzer

Oracle: Connecting to Linux with PuTTY

Oracle CD3: Manage Network

Final operational note After the network is configured, administrators connect to OCI VM private IPs through FastConnect. The OCI web portal is not in the SSH data path and is not required for routine PuTTY access.